1

复杂对抗环境下集群连通保持分簇路径规划

何运璐1   邱玉林1   林焕1   张宇2   王琳1   丁良辉1*

（1．上海交通大学集成电路学院  上海 200240  2. 92941部队 葫芦岛 125000）

摘  要  针对复杂城市对抗环境下现有人工势场类算法存在的路径震荡与死锁问题，提出一种结合全局路径采样与局部人工势场的通信连通保持路径规划算法。该算法通过构建分簇分层机制以降低势场耦合复杂度：簇间利用C-RRT*与Fréchet距离约束机制生成全局路径关键点；簇内基于最大SINR瓶颈生成树的局部人工势场法实时计算单步路径点。仿真表明，相比CMUFC 算法，典型场景下该算法平均到达时间降低29%，通信连通率提高17%。

关键词  多智能体路径规划，无人机集群，城市对抗环境，通信连通保持

引用格式  何运璐，邱玉林，林焕，张宇，王琳，丁良辉.复杂对抗环境下集群连通保持分簇路径规划[J].指挥与控制学报，2026.

Connectivity-Preserving Cluster Path Planning for Swarms in Complex Adversarial Environments

HE Yun-Lu 1  QIU Yu-Lin 1  LIN Huan 1  ZHANG Yu 2

WANG Lin1   DING Liang-Hui1*

（1．School of Integrated Circuits, Shanghai Jiao Tong University, Shanghai 200240, China

2．Unit 92941, Huludao 125000, China）

Abstract  To solve local minima and oscillations in APF algorithms under urban complex adversarial environments, a connectivity-preserving path planning algorithm combining global sampling with local APF is proposed. This method utilizes a hierarchical clustering mechanism to mitigate potential field coupling. C-RRT* with Fréchet constraints generates global inter-cluster waypoints, while an MBST-based local APF generates real-time intra-cluster trajectories. Simulation results show that the proposed method reduces average arrival time by 29% and improves communication connectivity by 17% compared with CMUFC.

Key words  multi-agent path finding, UAV swarm, urban adversarial environments, communication connectivity-preserving

Citation  HE Y L, QIU Y L, LIN H, ZHANG Y, WANG L, DING L H. Connectivity-preserving cluster path planning for swarms in complex adversarial environments[J]. Journal of Command and control, 2026.

近年来，由多架无人机（unmanned aerial vehicle,UAV）组成的无人机集群因其灵活性、高机动性及协同作业能力在遥感监测、城市巡检、通信覆盖以及军事侦察等民用和军用领域展现出巨大的应用潜力[1-4]。与单机作业不同，无人机集群在执行任务时不仅需要通过自主路径规划避开建筑障碍，更需要在信息空间上维持稳定的通信网络，以保障侦查信息的实时共享、协同打击、态势数据的回传以及指令的接收与响应等[5]。若缺乏可靠的通信链路而仅依赖机载视觉或本地感